Bore Water Test Report

Dear Sir

The following values are for the bore water samples you have given

| Parameter | Unit | 1st Sample | 2nd Sample | Drinking |
| --- | --- | --- | --- | --- |
| Ammonicalnitrogen | mg/l | 5.8 | 1.6 | Absent |
| Nitrite | mg/l | 2.9 | 0.8 | Absent |
| Nitrate | mg/l | 10.8 | 16.8 | < 20.0 |
| Fecal Coliforms | per 100 ml | 70.0 | 11.0 | Absent |
| Sulphide | mg/l | 2.0 | 0.5 | Nil |

The presence of the above parameters indicates the submitted water might have been contaminated by the sources as I said before, the rest of the parameters are within the limits.

This is for your kind information.

Yours truly,

S.L. Narasimha Rao

After testing your bore water, it was observed that the water has been contaminated by bacteriologically and chemically. Normally bore waters did not show the presence of coliform organisms and high ammonical nitrogen, nitrite and nitrate. The possible source of contamination might be from seepage of sewage/from surrounding drains or disintegration of dead animal. But contamination by seepage of sewage takes many days and depends on many factors like physical properties of soil and ground water chemistry.  This is purely my personal opinion. You are requested to take experts opinion from Geo engineering and environmental engineering fields regarding the source and remedies of contamination.

Meanwhile do not use this water for drinking, bathing, washing and cooking purposes unless it is treated and free from contamination.