Urban Waters

Background Material for

Arghyam’s second annual conference

April 4-5th 2007, Bangalore

Arghyam

Bangalore

www.arghyam.org

Table of Contents

1	Water in the Urban Context – An Overview	ii

2	Background	ii

3	The Fine Print	iii

4	The Peculiar Problems And Needs Of Slum Clusters	iii

5	The Poor: Capacity to Pay?	iv

6	Pollution	iv

7	Water and Sanitation: Inextricable Linkages	iv

8	Water Footprints: ‘Yeh City Maange More’	v

9	Impact Of Urban Excess Drawal On Other Uses Of Water	v

10	Groundwater	vi

11	Urban Group Housing: The Isolation Paradox and The Ratchet Effect	vi

12	System Efficiency and Coordination Issues	vi

13	Integrated Urban Water Management	vii

14	Who or What is a ‘Stakeholder’?	viii

15	Legal Issues	viii

16	Current And Projected Requirements	xiii

17	Best Practices: Learning From What Works	xiii

18	Conclusion	xviii

19	References	xix

# Water in the Urban Context – An Overview

There is a vast body of research on urban water issues; a bewildering number of organizations are working on the subject; and it has a wide array of dimensions. This overview seeks to present a snapshot, as simple and clear a picture of this totality as is possible while attempting to retain enough detail for it to be of interest and use as a starting point for discussion. The focus here is on India, while the larger field of vision is the developing world and then, beyond that, the world itself.

Some of the key questions being addressed here:

- What are the central facts of India’s urban water situation?

- What are the specific problems of our cities and towns?

- Who are the stakeholders in this picture?

- What are the equity issues?

- What is the legal framework?

- What are our current and projected requirements?

- What examples and best practices can we draw on from India and around the world?

# Background

By the end of this year, half the world’s population will live in cities [1]. India, which has always been said to live in its villages, is already close to 30% urban. This amounts to 286,119,689 people living in 5545 urban settlements [2], a figure that approximates the entire population of the US. Thus, if ‘Urban India’ was perceived as a distinct nation, it would be, on its own, the fourth largest nation in the world! Putting this in a global context, three of the world’s 10 most populated cities are in India: Mumbai (18.3 million), New Delhi (15.3 million) and Kolkata (14.3 million).

Of the 27 Asian cities with populations of over a million, Chennai and Delhi are ranked as the worst performing metropolitan cities in terms of hours of water availability per day, while Mumbai is ranked as second worst performer and Calcutta fourth worst [3].

There have been improvements, perhaps not always in terms of results but certainly in terms of an increasing focus on the issues of water in general and urban water in particular. Attempts have been made to relate these efforts to the Millennium Development Goals and this provides a certain direction and impetus to what was earlier an often ad hoc and disjointed set of initiatives. There is a growing awareness of the inter-linkages of different aspects of urban water issues as well as the links between these issues and the rural situation. But the actual picture is far from reassuring.

# The Fine Print

While the 2001 Census states that 90% of urban India has access to safe drinking water, only about 50% of this population is directly connected to the distribution system, and another 24% accesses public standpipes. And the 10% left out of the system refers to nearly thirty million people, which is about half the population of the UK. Even this information on access to water is unreliable, because it is generally based on aggregated statistics, equates installed capacity with actual service provision, leaves downtime and disrepair unaccounted and says nothing about the quality of water or the hours of supply.

Transmission losses are high. Where the Central Public Health and Environmental Engineering Organization (CPHEEO) allows for 15% unaccounted for water (UFW), levels in many Indian cities are as high as 40% or even 50%. This arises from both the poor condition of the infrastructure and unauthorized tapping of the supply. Cost recovery is poor and there is heavy cross subsidization. One study revealed that less than 300 class 1 and 2 cities have any metered connections. In many urban centers a large percentage of domestic connections are unmetered while in a little above one-fourth of the urban centers all non-domestic connections are also unmetered [4]. In most metros, people spend a considerable amount on water tankers, bottled drinking water and purification, alongside what they pay the municipality.

# The Peculiar Problems and Needs of Slum Clusters

Most Indian metros have large slum clusters that are not properly serviced with either water or sanitation. For example, 45% of Delhi’s population lives in unauthorized colonies, rural villages and slums and JJ clusters. The current number of slum dwelling units is estimated at about 0.6 million and the population living in slums and Yamuna clusters is projected at about 3 million. Out of the 0.6 million JJ units, nearly 62,000 units are estimated to be located in the river bed of Yamuna on both sides of its stretch along Delhi and on the embankments of a few major storm water drains such as Najafgarh drain, Barapulla drain etc. During dry weather these slum dwellers use open areas around their units for defecation. In this way, the entire human waste generated from these 62,000 units along with the additional wastewater generated from their household is discharged untreated into the river Yamuna [5].

Thirty thousand families live on Mumbai’s pavements. They do not have access to either water or lavatories because in the eyes of the law they are ‘illegal’ and should not be living there. Almost half of Mumbai’s 12 million people live in slums or dilapidated buildings. They are located on open land, along railway tracks, on pavements, next to the airport, under bridges and along the city’s coastline. Although there are variations, generally this half of the population gets little water and has even fewer lavatories. The water, when available, is often unclean. And the lavatories that exist are usually filthy, broken down and generally unusable: it is safer to defecate in the open than to use some of them. So few are functional that open defecation is the only alternative for millions of people.

# The Poor: Capacity to Pay?

Much has been made of the idea that the urban poor actually have a capacity and a willingness to pay for water, but this hides certain important facts. First, if they are in fact paying for non-municipal sources like bottled water and tankers, it is not because they are flush with funds; it is sheer desperation and the lack of an alternative, and it cuts into their household expenditure in a major way. In the town of Dehradun in Uttaranchal for example, poor households were found to be paying up to five times the prevailing water rate in coping costs. [14] Second, what one is effectively saying is that one should cease to look at ways to subsidize water or provide something beyond public water access points for the poor, who are the one category of people who most deserve these subsidies.

# Pollution

Water supply to the cities comes from surface bodies, groundwater and precipitation. Major rivers like the Ganga and the Yamuna are struggling to cope with the huge volume of waste being dumped into them from various sources. The UN World Water Development Report has categorized India among the worst countries for quality of water, as well as ability and commitment to improve the situation. This report compiled by 23 UN Partners of the World Water Assessment Programme (WWAP) also states that Indian rivers have approximately 20 times more lead than those of the industrialized countries. [18]

Chemical contaminants, namely fluoride, arsenic and selenium pose a very serious health hazard in the country. It is estimated that about 70 million people in 20 states are at risk due to excess fluoride and around 10 million people are at risk due to excess arsenic in groundwater. Apart from this, increase in the concentration of chloride, TDS, nitrate, iron in ground water is of great concern for a sustainable drinking water programme. With over-extraction of groundwater the concentration of chemicals is increasing regularly. Ingress of seawater into coastal aquifers as a result of over-extraction of ground water has made water supplies more saline and unsuitable for drinking. Pollution of ground and surface waters from agrochemicals (fertilizers and pesticides) and from industry poses a major environmental health hazard, with potentially significant costs to the country. The World Bank has estimated that the total cost of environmental damage in India amounts to US$ 9.7 billion annually, or 4.5%of the gross domestic product. Of this, 59% results from the health impacts of water pollution (World Bank 1995).

# Water and Sanitation: Inextricable Linkages

Water shortages and lack of infrastructure affect sanitation as well. Some 63.5% of urban Indians lack access to toilets. Estimates of access to excreta disposal systems in urban areas vary from a low 48 percent to a high 70 percent. Two thirds of the Class 1 cities and four fifth of the small urban centers don’t have sewerage systems. A survey of 345 towns with populations between 50,000 and 100,000 revealed that over 95% of them do not have any wastewater treatment facilities. [15] Even in the ones that have systems, coverage is partial. Put another way, a study by the Central Pollution Control Board in 1994-95 shows that the total waste-water generated in 300 Class-I cities is about 15,800 million litres a day (mld), while the treatment capacity is hardly 3,750 mld. [6]

In the 23 metro cities, over 9000 million litres of sewage is generated daily, of which about 60% is generated in the four mega cities of Mumbai, Kolkata, Delhi, and Chennai. [17] Of the total wastewater generated in the metro cities, hardly 30% is treated before disposal. Most of even these metro cities have only primary treatment facilities. Thus, the untreated and partially treated municipal wastewater finds its way into water sources such as rivers, lakes and ground water, causing water pollution.

# Water Footprints: ‘Yeh City Maange More’

The water footprints of cities are getting larger, but urban water allocation from interstate rivers is a contentious issue and new supply projects are in greater risk of not receiving supplies. The Sonia Vihar water project in Delhi is a case in point. Not only has the project run into the kind of cost and time overruns that are characteristic of most Indian infrastructure projects; even now, when it is more or less complete, it is far from being a reliable solution to the problem of augmenting water supply to Delhi. At any given time, Uttar Pradesh can shut the tap, citing any one of a number of reasons, as for example seasonal decreases in water flow affecting its own requirements. Moreover, the minute you have a scheme that transports water from one state to another, it becomes a victim of interstate politics in regard to matters that may have nothing to do with water itself. Water becomes a bargaining tool, a basis for quid pro quo. Drawing on external sources in a major way to supply urban requirements also raises equity issues. What it says is that the city or town has greater priority than the surrounding areas. This, in a context where little or nothing is being done towards improving efficiency or curtailing and rationalizing urban demand.

# Impact of Urban Excess Drawal on Other Uses of Water

All this is to say nothing about the impact of reducing peri-urban or rural water availability as a consequence of supplying to the urban areas on irrigation and agricultural productivity. As an example, over the last five years the farmers of two rural districts outside Chennai have started selling water from their irrigation wells to the city's water utility. This, to the detriment of cultivation in their own lands and those of neighbouring farmers. In villages like Velliyur, Vishnuwakkam, Magaral, Selai, and Kaivandur, Chennai Metrowater extracts ground water 24 x 7 with as many as 30 ten-horsepower pumps. With dwindling groundwater, the decline in agriculture is already apparent in the last two years that the water utility has started pumping from private irrigation wells. While the number of farmers selling water in the village of Magaral has increased from 12 in 1993 to 22 in March 2001, the duration has also increased from 12-hours a day for four months in 1993 to round-the-clock, round-the-year in 2001. [16] A clear vicious hierarchy of priorities is emerging: metro cities on top; smaller cities a poor second, slums within cities third, and rural areas a poor fourth.

Conversely, there is a hidden potential for increasing urban water supply vis a vis systems that access rural water for the cities. If agricultural irrigation were more efficient, it has the potential for releasing vast amounts of water for urban use, because the relative quantum of water used for irrigation is so much greater than that needed for urban supply. But this runs up against political barriers.

A case has been made recently for implementing the idea of a water envelope, that is to say that a city or town should have a limit set on its water supply allocation and be required to manage within it (with, of course, adjustments for population increase). This would be an enlightened solution, though perhaps not an easy one to implement. The question is increasingly however not one of ease of solution but necessity.

# Groundwater

Groundwater extraction has emerged over the years as a major component of water supply and it is easy to see why (here one is talking of extraction by individual consumers). When the municipality fails to supply adequate water and the consumer knows that this precious resource is available, literally below his or her feet, the temptation to sink individual bore wells is too strong to resist, especially when legal restrictions are ambiguous and even these are not enforced. It is so much easier than applying active citizenship principles to get the municipality to deliver.

Groundwater extraction by individuals is supplemented by that done by companies either supplying water or extracting it for their needs, which is obviously a much larger scale of activity. It is largely unregulated, is far higher than replenishment, and urban water tables are dropping alarmingly. With a few exceptions, urban rainwater harvesting schemes have not been pursued vigorously. Where they are pursued, it is mostly in the case of institutional buildings. Getting members of group housing societies to cooperate in installing such schemes goes against the strong prevailing mindset that demanding water is more important than managing it.

# Urban Group Housing: The Isolation Paradox and The Ratchet Effect

In urban group housing, a peculiar ratchet effect-type phenomenon emerges even where there is a reasonable supply of water to start with. One resident of a building decides that the supply is not enough for his/her needs or wants, and decides to install an on-line booster pump. This naturally reduces the availability to the others. They are forced to do likewise. It becomes a tussle between whose water pump is more powerful. Then they go to the next stage, which is to (illegally) increase the diameter of the pipe supplying water to their flat, thereby further increasing their share. The process is endless. So you have a situation where managing within available supply is seen as positively dangerous, because the perception is that your neighbour will then take advantage and get the better of you.

# System Efficiency and Coordination Issues

It is not merely poor infrastructure and lack of funds that are coming in the way of providing adequate water to our cities and towns. Inter-agency coordination (or lack of it) is another major hurdle both in operational efficiency and planning. Urban water in its various avatars is handled by a multiplicity of agencies….

The Municipal corporation handles water supply and sewerage, maintenance, stormwater drains and the like; the water supply utility sets up water supply infrastructure; the lake development authority (where lakes are a source of urban water, as in Mumbai) is in charge of urban water bodies; the groundwater department is usually within another department/authority like Dept of mines and geology, and is responsible, as is obvious from the name, for groundwater (but it has a more geological/technological orientation than one of service provision); then you have the CBOs and NGOs that voice the demand, especially that of the urban poor (here there has traditionally been either a disconnect or even an antagonistic relation vis-à-vis the state agencies, though this is slowly changing); and then you have the pollution control board, which handles water quality testing and enforcement.

In the case of some metropolitan cities, attempts are being made to integrate the functions if not the organizations themselves, but there is a long way to go. It’s a case of the right hand not knowing (in a negative sense) what the left hand is doing (or rather, since this is India, the many hands of the water goddess working independently!)

# Integrated Urban Water Management

In this context, Integrated Urban Water Management (IUWM) is a vital need. IUWM is the practice of providing all stakeholders with a vision, framework and comprehensive set of practical actions to enable the holistic, sustainable and equitable management of freshwater resources. The concept of IUWM embodies the various components of water management, including the political, environmental, social, economic and technical. It is a bold and challenging approach, but one which encourages optimism for the development of broad-based solutions to the many diverse water-related challenges experienced in urban and peri-urban areas. In the Arghyam vision, an effective framework for IUWM must rest on the four pillars of sustainability; environmental, financial (economic), technical and institutional, as well as the additional pillar of social equity. This means that attention must be given to two fundamental and interconnected challenges; ensuring sustainable urban water catchments and delivering equitable access.

The first challenge of IUWM is developing and implementing systems to support the integrity of the water catchment area and the resources within it. In an urban setting, the water catchment area includes water resources; underground water and surface water such as rivers, dams, rainwater, lakes and tanks. It also includes the man-made networks of water distribution channels, storm water drains and sewerage infrastructure. The flows between and within these water resources and man-made networks is the urban hydrological cycle, in essence it is a system of systems. The connectedness of these systems means that if the integrity of any part is compromised then the sustainability of the entire system will be threatened.

The second urban challenge relates to access of water. In urban areas distribution of water services and resources is inequitable in both quantity and quality. The urban poor are not connected to formal connections and suffer from insufficient quantities of easily accessible safe and affordable water. Often the poor are forced through poverty to use contaminated water. Where connections do exist, water pressure may be so low that women spend far too many of their productive hours waiting at the tap. The social and economic impacts of this are immediate and intense impacting on gender equality, education, health and livelihoods. By it also perpetuates cyclical poverty and the capacity of the poor to live healthy, dignified lives with opportunities and options.

These two challenges are not discrete, they are intimately connected. Take the following scenario as an example. Sewerage infrastructure in the city is aging and desperately needs repair, but there has been no investment to upgrade the pipes in this part of town for many years. As a result untreated waste that should flow to a treatment plant seeps into the underground water system which becomes contaminated and unfit for consumption. People continue to drink the underground water. For many urban poor it is the cheapest and most easily accessible water source, but it makes them and their children sick…

Unfortunately this is not uncommon in our cities, and it is only one brief example of how our current systems are failing. Where did the problem begin? Perhaps it was a lack of financial resources. Maybe it was that the poor household had limited options. Or possibly there was a lack of knowledge sharing and planning between the institutions responsible for sanitation, underground water and essential services for the urban poor. Probably it is a combination of these factors as well as many more.

We know that our urban areas are continuing to grow at a rapid rate and that as they do the dual challenges of sustainability and equity reach critical levels. Solutions to both these challenges are possible but rest on ‘getting right’ the institutional arrangements in place to govern and manage resources at all levels of society, from government to household. This means tackling the tough issues of capacity building, political accountability, poverty, financing, and tariff structures in an integrated way. The IUWM approach to understanding and managing our natural and man-made urban water resources is not optional; it is an immediate necessity if we can hope to imagine a future with water secure cities.

# Who or What is a ‘Stakeholder’?

In this context it is worth going into the notion of stakeholders as well. The word ‘stakeholder’ has an interesting history. It comes from the days of prospecting for gold mines in the American wild west. A stakeholder, in the traditional sense is someone who stakes an ownership claim over something. Today however the word has come to mean anyone who is involved in a particular sector, whether as financier, administrator or user/beneficiary. There is a need to separate these sub-definitions lest everyone from multilateral organizations to slum dwellers are seen as equals in a proprietorial sense.

# Legal Issues

Water Law is a field that has many gray areas. It ranges from issues of constitutional provisions to municipal regulations at the micro level. We lack provisions to safeguard the source of water. For example, where traditional tanks are storing or providing water, if in three seasons those tanks do not (for whatever reasons) collect water, they can be declared as wasteland and destroyed to be used to build housing or other infrastructure. A new urban dimension is emerging with respect to riparian rights. Traditionally, these are seen as rights over the use of water from rivers and canals that run through a geographical area. Increasingly, at a popular level, people are also questioning why they should not have access to the water that runs through major pipelines carrying water through their areas to distant urban agglomerations – or rather, not just questioning it, but taking ‘informal action’ to access it by breaking into the pipelines.

We have norms for prescribing appropriate minimum quantities of water, but these norms are mostly used only in the context of statistical surveys of actual situations rather than being converted into rights. The makers of the Constitution provided definitions for various kinds of citizens’ rights, but at a time when the current extreme water scarcity situations were not envisaged, rights to water were not included in any meaningful sense.

The vexed issue of ownership is yet to be tackled: does one look at water as a common resource, an economic good or private property? There is a very mixed (and mixed-up) situation in this regard. Plots of land can be demarcated and ownership assigned, but obviously the groundwater beneath them cannot be separated in a similar fashion because what one person draws from the land beneath his/her plot affects all those in the vicinity.

There are also peculiar legal or definitional situations that arise with regard to the word ‘urban’. In Tamil Nadu for example, several previously rural areas were declared urban and a situation emerged where the state seemed to be even more urban than Maharashtra. But when it was found that being urban meant being denied developmental funds available for rural areas, several of these new urban areas were denotified!

Regulatory authorities are still on the drawing board in many cases; The Maharashtra Water Resources Regulatory Authority is the only one in the country that is constituted in the appropriate comprehensive form for it to perform its tasks. Kolkata is the only example of a place that had set up District Committees along the prescribed lines. Elsewhere much has to be done towards creating and empowering integrated regulatory bodies.

## Water and the Indian Constitution

Water resource management is a State and concurrent subject under schedule VII of the Indian constitution. Water finds a mention in several sections of our Indian Constitution. Specifically,

- Water is included in the Constitution in the State List as Entry 17 of List-II. This entry is subject to the provision in the Union List of the 7th Schedule, Entry 56 of List-I.

- Parliament empowered to make laws, for adjudication of disputes between states on interstate rivers on river valleys (Art.262) (Union list; entries 25, 34,56,57,58 and 97; concurrent list; entries 20 and 30; statelist; entries 5,13,17 and 21)[7]

- Water is also mentioned as a fundamental duty: Citizen’s duty to protect and improve the environment [Art.51A(g)]

- Water in the Directive principles of the State policy: While there is no direct reference here, it is deemed to have been included in the State’s obligations i.e. to endeavor to protect and improve the environment (Art. 48A) and to secure distribution of ownership  and control of material resources ( natural resources of which water is a part) of the community to subserve the common good and to operate the economic system not to result in concentration of wealth and means of production to the  common detriment [ Art.39(b) and (c)]; [8]

- Local Self Government - State legislatures may empower the local self government institutions like panchayats (Art.243G) and Municipalities (Art 243W) to administer and implement laws concerning management of local resources. These include minor irrigation, fisheries, water management and water shed development; drinking water and maintenance of community assets etc.(Schedule XI and Schedule XII)

There are numerous institutions dealing with water both at central and state level like Ministry of Water Resources, Ministry of Urban Development, Ministry of environment & Forests, Central and State Groundwater Boards, Ministry of Agriculture, Ministry of Industry, State Irrigation Departments and Central and State Pollution Control Boards.

## Policies

The ideal method of law and policy-making involves law to have its root in a felt need. The ideal situation of the system of governance involves the policy, the law, the institution of implementation, the plan and programme of action and actual implementing in a sequence and ordered form.[*]

### National Water Policy

The success of the National Water Policy depends entirely on evolving and maintaining a national consensus and commitment to its underlying principles and objectives.

(Excerpts from the National Water Policy 2002)

- The National water policy 2002 has elaborated the need for a policy for water. [9]

- It recognizes water as a precious asset-as a resource it is indivisible system and an integral part of the ecosystem.

- Encourages efforts to develop, conserve, utilize and manage water in an integrated and sound basis, with a national perspective and keeping in view the need of the states concerned.

- Planning and development of water resources development project as multipurpose projects

- Require proper maintenance and constant vigilance over safety of structures and system, water quality and a periodic assessment on a scientific basis of the ground water potential, utilization and development.

- Priorities and water allocation in planning and operation of systems in drinking water, irrigation, hydro power, navigation, industrial etc.

### State Water Policies

The National Water Policy states that in order to achieve the desired objectives, State Water Policies, backed with an operational action plan, and shall be formulated in a time bound manner.

An example of State Water Policy, the Rajasthan State Water Policy is briefly explained here.

The state of Rajasthan is one of the driest states of the country with only 1% of the country’s water resources. The State water policy has been drafted to overcome the highly uneven and inequitable distribution of water both in time and space. The policy covers every aspect of water like ground water development and management (legal, organizational, social, technological, educational, and environmental), watershed management, water zoning etc. To improve and streamline their scope and cover in the legal framework all aspects pertaining to water resources management, flood protection and drought proofing, licensing, water rights etc have also been taken care of. [10]

## Legislation

Environment protection in India has a 5000 year history - for example, Kautaliya had set different punishments for people cutting different parts of the trees. In India today, there are several laws enacted for the protection of environment with water related laws being a major focus. [11]

### Existing National Laws

Some of the existing laws are listed below:

- The Water (Prevention and Control of Pollution) Act 1974,

- Water Cess Act 1977, Environment (Protection) Act 1986,

- Environmental Notifications,

- Environmental Audits,

- Ambient Water quality Standards,

- Transfer of Property Act ,

- Tribunals for interstate disputes,

- The interstate Water dispute Act 1956,

- National Water ways Act 1985.

A few other legislation exists regulate activities to certain rivers like Damodar Valley Corporation Act 1948.

### Proposed Legislative Efforts-:

Model Bill to Regulate and Control the Development of Ground Water 1992, modified 2005) -: This bill vests all water rights in the State, establishes a command and control regime for ground water regulation, empowers state governments to designate problem areas (like overdrafts) and proposes to establish ground water management authorities. This is currently in circulation among State Governments. The modified Model Bill 2005 states that rain water harvesting should be used as an important tool for ground water recharge.

### Existing State Laws-:

There have been piece-meal and sporadic legislative efforts dealing with different aspects of water management including irrigation, drinking water supply, floods control, pollution, fisheries etc under a host of authorities across different states. [12]

Some good examples of State Laws are the U.P Bhoomi Evam Jal Sambhav Adhiniyam, 1963, and the Andhra Pradesh State Policy effort in attempting to deal with water and land. Proposals to enact ground water conservation laws are under active consideration in some states like Karnataka.

### Analysis of the Water related laws-:

Although elaborate provisions of water laws appear to exist, the law remains ineffective and the environment continues to deteriorate. Experts opine that principal legislation is repetitious and poorly drafted .The laws are not backed by sound policy pronouncements and legislation does not appear well thought but seems ad hoc. [13]

- The Pollution Control laws are based on command and control regime with an emphasis on punitive rather than pro-active and preventive measures. Though the boards have wide ranging powers, including the ability to hold industrialists personally liable for environment damages, it is rarely exercised.

- Some areas like groundwater pollution remain outside the purview of the pollution control laws. An example is The Transfer of Property Act where ground water is attached to land and no limitation exists on how much ground water a particular land owner may draw.

- There is no comprehensive national law dealing with every aspect of water management (access, distribution, use, protection and conservation).

- While Water (Prevention and Control of Pollution) Act, 1974 refers to maintenance and restoration of wholesomeness of water as its principal objectives, the operational aspect of law is confined to the sector of water pollution control only.

- Legislative efforts have paid inadequate attention to conservation, recharge and maintenance of water quality.

- Citizen’s Rights and Responsibilities in relation to water is legislatively viewed, more as something to do with owner of the land or as some interest holder, rather than as a “trustee” of a resources having an obligation in an intergenerational sense.

- A decentralized and community based approach is missing.

- Water conservation and maintenance of quality of water have drawn least attention among legislative efforts by the States.

- Expression of water in the EPA 1986 does not have much emphasis on water issues.

# Current and Projected Requirements

Urban water requirements are expected to double from 25 billion cubic metres (BCM) in 1990, to 52 BCM by the year 2025. A joint 1998 report by UNICEF and World Wide Fund for Nature (WWF) predicts that India is going to be water stressed by 2017. This stress will be of a heightened nature in the urban areas. Urban conglomerations are water intensive by their very nature. Lifestyles adopted by their inhabitants (piped water supply, flush toilets etc.) only add to the woes. There are indirect urban water factors as well. Urban dwellers eat more meat and water intensive crops: for example, each kilo of wheat requires around 500 litres of water, a kilo of rice upto 2000 litres, while a kilo of meat requires around 20-50 thousand litres of water. Automobiles, the hallmark of urban centres, have production processes that are highly water intensive. The production of one car consumes directly and indirectly about 400 cubic metres of water. These are but a few examples of the rapidly increasing consumptive nature of our cities and towns.

Larger cities, however, have greater capacity and access to funds. It is the small towns that are in greater need of assistance to provide adequate water to their residents. Many of these towns are not able to meet even the minimum norm of providing 70 litres per person per day to their residents who rely heavily on informal sources such as hand-pumps, wells and ponds. Small towns are also often rural in character and hence have water needs related to animal husbandry and agriculture which create additional pressure on demand. Vagaries of weather add to the problems of these towns where water sources may dry up and financial constraints prevent them from initiating new schemes.

The CPHEEO has estimated that the funds required for 100 percent coverage of the urban population under safe water supply and sanitation services by the year 2021 is Rs.172,905 crores. Towards this end, there has been an evident increase in the focus on water in the Ninth and Tenth Plans and several schemes have been initiated over the years with varying results. Schemes such as the Accelerated Urban Water Supply Programme have been initiated and more recently, The Jawaharlal Nehru National Urban Renewal Mission (JNNURM) has provided a further impetus to these efforts. Some work has been done towards assessing the impact of these schemes, but much more is required. The impact of the JNNURM in particular, since it is the most recent, will only be evident with time.

# Best Practices: Learning From What Works

There is a vital need to study best practices available from all over the world, (including those being followed in parts of India itself) and to see the extent to which they can be adapted and applied to our situation.

There are examples of best practices in all aspects of urban water management, be it water supply, sanitation, wastewater management, services to the poor etc Fortunately, this is an area where a huge amount of information is available courtesy of both multilateral organizations and NGOs/CBOs. It is beyond the scope of this document to make a comprehensive list so just a few examples have been mentioned below.

## Institutions:

It is important that all the water in a city is managed by the same institution. This institution should be responsible for sourcing the water, treating it, supplying it. Further, the wastewater generated also needs to be managed by the same institution that supplies water. This wastewater could be treated and recycled by the institution and the treated water supplied through a different pipeline to consumers. It could also be allowed to flow into waterbodies in the city, from where it could recharge into the ground.

Two good examples of water management institutions are mentioned below:

The Bangalore Water Supply and Sewerage Board (BWSSB) is among the best performing water supply institutions in India for some of the following reasons:

- All legal connections are metered.

- It convenes a Water Adalat once in a month. These Adalats settle billing and any other disputes connected to water, sanitary, non-receipt of bill, irregular meter reading etc. They are convened at the sub-divisional level and are chaired by the concerned Executive Engineer.

- The system has a progressive block tariff so that the consumers are incentivised to consume less. The domestic tariff rates are mentioned below.

Domestic Tariff

| Consumption Slab Litres | Tariff (Rs./Klit.) |
| --- | --- |
| 0-8000 | 6.00 |
| 8001 – 25,000 | 9.00 |
| 25,001 – 50,000 | 15.00 |
| 50,001 – 75,000 | 30.00 |
| 75,001- 100,000 | 36.00 |
| 100,000 and above | 36.00 |

For more information, visit http://bwssb.org/.

Sydney Water is a statutory State owned corporation, wholly owned by the people of New South Wales, Australia. Sydney Water provides drinking water, recycled water, wastewater services and some stormwater services to more than four million people in Sydney, and the surrounding areas of Illawarra and the Blue Mountains. Some elements of Sydney Water’s good practices are:

- Sydney Water collects and treats more than 1.2 billion litres of wastewater each day.

- Wastewater collected in the sewerage system flows to Sewage treatment Plants (STPs) where it is treated before being reused or discharged to rivers or oceans.

- Sydney Water recycles almost 42 million litres of wastewater a day.

- All biosolids captured (100%) are used for agricultural or horticultural purposes.

- Sydney Water also operates and maintains stormwater pollution control devices such as gross pollutant traps, litter booms and sediment traps. Every year, Sydney Water's stormwater pollution control devices collect about 2,150 cubic metres of rubbish and 2,200 tonnes of sediment.

For more information, visit www.sydneywater.com.au

## Community Involvement:

Gramalaya, an NGO has been working extensively in sanitation in urban and rural areas of  Tiruchirapalli district. Among their achievements is what is touted as the country's first sanitised slum, Kalmandhai, in Trichy city. Community toilets were built for men and women and are run at a nominal usage charge, which returns enough money for the ongoing maintenance. The toilets are maintained by Self-Help Groups (SHG) consisting of women in the slum. There is also a separate children’s toilet area. With the availability of affordable toilet facilities as well as strong grassroots social pressure, open defecation has been completely stopped. Three to six hundred people use any given community toilet daily.

Gramalaya conducted a public toilet survey in December 2001 in 182 locations of Trichy city limits. The findings have motivated the City Corporation to allocate more toilet units to the SHG pay-and-use system.

A special effort that has been made is building child-friendly toilets (CFTs) for small children. The models were designed locally. CFTs have been built in different slums in Trichy and 235 children have been using them on a daily average.

For more information, visit www.gramalaya.org

The Society for the Promotion of Area Resource Centres (SPARC), Mumbai is also supporting similar efforts in Pune, Mumbai, Hyderabad, Bhubaneshwar, Bangalore and several smaller towns in India. They have also implemented community toilet approaches and designed the physical models for normal and children’s toilets. The cost of construction per toilet seat is comparatively low, as community contractors are employed. In Pune, SPARC is working on a plan to provide complete sanitation cover for Pune over a period of five years.

For more information, visit http://www.sparcindia.org/

## Standards:

The water that is supplied to the community must conform to the drinking water standards set by the law. To ensure that the agency supplying the water complies with these norms, regular reviews need to be conducted by the competent authority.

Since the quality of water supplied depends heavily on the quality of the source water, source-protection is important. Stringent laws and their compliance are required to ensure that wastewater cannot be discharged into such sources directly.

An example of this is the system followed by the German Water Industry.

Some salient features of the system followed by the German Water Industry are:

- In Germany the public drinking water supply must comply with the German Drinking Water Ordinance and the European Drinking Water Directive. This compliance is nationwide. The competent health authorities of the municipalities and administrative districts review their compliance.

- The German law provides a binding imperative to minimize chemical substances in water. In the delivered drinking water in Germany for the heavy metals cadmium, mercury, lead, chromium, antimony and nickel has been measured in the recent past.

- The quality of drinking water depends to a great extent on the quality of the abstracted raw water. This is ensured through more than 17,584 water protection areas. In these areas, the requirements exceed the normal nationwide waterbody protection levels.

- The criteria for the quality of water supply are in compliance with the minimum supply pressure.

The degree of connections to the public water supply in Germany is 99%.

For more information, visit www.dvgw.de/pdf/branchenbild_en.pdf

## Tariff:

In South Africa, the Free Basic Water (FBW) for residential customers is a government-subsidised tariff system to ensure basic human needs are always met. Some of the salient features of this system are:

- Block tariffs have replaced the flatrate method of charging the customers. This means that the consumer pays for as much water as he uses.

- Since water has been recognized as a basic human right, 6000 litres/person/month is made available free of cost. The FreePay meter options allow for dispensing of 200 litres per day or 6 000 litres per month.

- Water and sanitation services are now combined. Tariffs are based on a volumetric charge which allows for the provision of Free Basic water and Free Basic sanitation.

The charges for water supply are as follows:

| Consumption in Kilolitres | Cost in Rands (1USD =7.25Rand) |
| --- | --- |
| 1 to 6 kl | Free |
| 7> to 10 kl | 4.20 |
| 11> to 15 kl | 5.60 |
| 16> to 20 kl | 7.00 |
| 21> to 40 kl | 8.40 |
| 41+ | 9.90 |

For more information, visit www.johannesburgwater.co.za

## Wastewater Management:

DEWATS stands for “Decentralized Wastewater Treatment Systems”. DEWATS is a technical approach to reduce water pollution by small and medium enterprises (including apartment complexes, hospitals and more) and settlements in densely populated areas. It has been designed by Bremen Overseas Research and Development Association (BORDA) of Germany. Some of the benefits of DEWATS systems are:

- Efficient performance

- DEWATS is efficient in removing BOD of the wastewater by more than 90 percent. This makes the water safe for non-potable use.

- Non-dependence on energy

- This system does not use electricity at any stage as water flows from one unit to the other by gravity.

- Reliability and longevity

- Unlike septic tanks, which get filled up fairly fast (especially in the situation of community toilets), DEWATS system can be used continuously with desludging once in two years.

- Tolerance to inflow fluctuation and peak loads

- DEWATS is designed keeping in mind the fluctuations in the inflow (like during a festival, in the area, the inflow from a particular community toilet might be high), so that it works efficiently at all times.

- Minimal maintenance

- There is no maintenance required in terms of regular cleaning of ‘registers’ (openings like manholes).

- Odourless operation

- Since the process takes place in closed containers under anaerobic conditions (in the absence of air), no odour is generated.

- Spatial requirement adjustable through underground construction

- In places where there is not enough open space available for construction of DEWATS tanks, space can be adjusted by constructing the system under the ground.

There are more than 250 DEWATS units running around the world including Bangalore and Tirucharapalli in India.

For more information, visit http://www.borda.de/conrat/Dewats_Mai05_72.pdf.

Singapore is a successful model of water management. Some key features in this model are:

- A single utility manages water sourcing, supply, stormwater and wastewater.

- Catchment areas of the city state where drinking water is sourced are restricted from being used for pollution-generating activities.

- Singapore has started cleaning wastewater to ultrapure levels. The resultant water is being used by commercial facilities like electronics plants with high-purity water requirements, as well as being bottled and sold and also poured directly into the reservoirs that supply Singapore's water. Singapore's plan is to ultimately meet 30% of its drinking water requirements from this treated effluent water -- called Nuwater. The cost of water generated this way is .30 S$/m3

- Desalination is also being developed as a water source. One desalination plant is currently running, with the cost of water generated being .78 S$/m3

- There is no cross-subsidisation between commercial and domestic users. There is an additional Water Conservation Tax to reinforce the water conservation message, and a separate statutory charge to fund maintenance and new construction. Between 2000 and 2004, the overall water requirement for Singapore actually went down, chiefly as a result of these demand management measures.

- There is a targeted subsidy for poor people, rather than ‘lifeline water’.

For more information, visit http://tinyurl.com/2jzckt

New York City is an example of good demand management of water. In 1991, the City launched a pilot water conservation program to stem rising demand. The program offered free leak detection and installation of water-saving plumbing devices such as low-flow showerheads and faucets, aerators, toilet tank displacement bags, and low-flow toilets. These services were provided to 10,000 homes with 1–3 families citywide. Starting in 1993, a larger scale water conservation program conducted leak detection for some 8,000 homes with 1–3 families and 80,000 apartments. The City provided an expanded range of water-saving showerheads and toilet devices, new outreach and public education, and energy conservation in cooperation with the electrical utility Consolidated Edison. The City has developed an audit of leakage as a basis for estimated long-term benefits from subsidized water conservation measures. By 1995, in-city average demand had dropped to about 1,300 mgd. Much greater savings are anticipated from continuation of leak detection and plumbing retrofit efforts. It is anticipated that one-third of the City's residential toilets will be replaced with 1.6-gallon-per-flush units by 1998.

A new law mandates the use of water-saving plumbing fixtures in all new construction or replacements of existing fixtures, including limits of 3 gallons per minute for sink faucets and showerheads and 1.6 gallons per flush for toilets. The State expects that this requirement will save over 500 mgd statewide.

For more information, visit http://tinyurl.com/2a2y4h

# Conclusion

Lastly, one must say that the entire picture of urban water requirements is linked not just to infrastructure, finance, governance and the like, but to urban developmental choices. If we are going to go in for cities which have large numbers of highly water-intensive constructions such as shopping malls, or for increasing the consumption and hence production of soft drinks, for example, and go in for them because we want to look like a ‘developed country’, we are obviously in for trouble. Developmental choices will have to be taken into consideration in a major way when we speak of urban water. Perhaps the most fitting conclusion would be to say that it is no longer a matter of ideology; no longer a matter of whether privatization is good or bad and so on. There is a stark question before us: when we talk of 8% growth (and remember, urban India contributes 50% of our Gross Domestic Product (GDP), have we realized that of all the necessary inputs for that growth, water may become the one that makes complete nonsense of it?

# References

- A report by the UN Commission on Population and Development, presented by UN Secretary General Kofi Annan to the UN Social and Economic Council on February 16.

- Census of India 2001.

- Background Paper - International Conference on New Perspectives on Water for Urban & Rural India - 18-19 September, 2001, New Delhi.)

- Raghupathi, 2005 (data are for 1999-2002)

- Yamuna Action Plan (http://yap.nic.in/delhi-slums.asp) .

- National Seminar on Integrated Solid Waste Management in India www.sai.uni-heidelberg.de/abt/home_uk/SWM_Workshop(8.9.2006).pdf

- The Constitution of India.

- The Directive Principles of State Policy

- National Water Policy 2002

- Rajasthan State Water Policy

- Kautaliya www.fordham.edu

- * CEERA, NLS, Bangalore

- Environmental Governance -1,Indira Gandhi Institute of Development Research

- ADB: Helping India Achieve 24x7 Water Supply Service by 2010

- Central Pollution Control Board,1989, Status of water supply and Wastewater Generation Collection Treatment & Disposal in Class II Towns 1988, Control of Urban Pollution Series:CUPS/31/1989-90

- GAMBIEZ M., LACOUR E. : Rural impact of farmers selling water to Chennai Metropolitan Water Board: a case study of Magaral Panchayat – 2003

- Central Pollution Control Board,1994-95

- The UN World Water Development Report - Water for People, Water for Life, 2003